TH-9800:

1) Connect the TH-9800 and PC via cable, then open the programming software to read the radio.

2) When the reading process is completed, click CtrL+F11, and input the password “9800TYT“. Then selects the frequency range you needed under the “Frequency range”.

3) Then write and save the setting into TH-9800.

4) Done!

TH-9800 Maximum Frequency Range:

- 26-33MHz (RX/TX) (AM band RX)

- 47-54MHz (RX/TX) (AM band RX)

- 136-174MHz (RX/TX)

- 400-480MHz (RX/TX)

TH-9000D：

1) Connect the TH-9800 and PC via cable, then open the programming software to read the radio.

2) When the reading process is completed, click CtrL+F11, and input the password “9800TYT“. Then selects the frequency range you needed under the “Frequency range”.

3) Then write and save the setting into TH-9000D.

4) Done!

TH-9000D Maximum Frequency Range:

- 136-174MHz (RX/TX)

- 220-260MHz (RX/TX)

- 400-490MHz (RX/TX)

TH-UV8000D/UV8000E:

- Press [U/V] key to turn on the radio, input the password “228228”, and press [F] to login.

- Then the top row will show “400->520”, and there are 3 options for selecting: a. DEFAULT, b. TABOO, c. CUSTOM, and press [∧] or [∨] to setup the range.

- DEFAULT refer to the Default frequency range, press [F] to save the setting.

- TABOO refer to Disabled frequency range, it’s unavailable when you use this range, you can ignore this part. Press [F] and go back. (TABOO is the range of 200-260MHz)

- CUSTOM refer to Custom range, press [F], then the bottom row will show “--- -> ---“, then you can press the number key to input the frequency in the limited range.

Eg. When you set up the range “420->480”, it means 420-480MHz is available in the range of 400-520MHz.

Compatible with the radio model MD390/MD9600/MD-UV380/MD-UV390/PD-500

- Install Radioddity.exe

- Connect the radio and PC via programming cable, turn on the radio and open the software Radioddity

- Select the model you want to spread the frequency.

- Then click “Read”

- Choose the frequency range you need under Frequency Range

- Click “Write”, the setting will be saved. Then you can test the radio after re-turn on the radio.

MD-380：

- Install MD-380.exe

- Connect the radio and PC via programming cable, turn on the radio and open the software  Factory

- Click “Read”

- Choose the frequency range you need under Frequency Range

- Click “Write”, the setting will be saved. Then you can test the radio after re-turn on the radio.