iRaddy GM Series Programming Guide

- Install and run the programming software.

2. Turn on the GM series radio, connect the radio and PC with the programming cable.
Set → Communication Port → select the corresponding port (the black one, not the default one)

- For the first time using the programming software, you can read frequency first by clicking the icon or by clicking Program → Read Data

Attention:

The following picture with a warning may pop out due to

①. The radio is turned off. (please turn on the radio)

②. The communication port number is incorrect. (please select the correct port)

③. The radio is in standby mode and the intercom screen is not lit. (please make sure the screen is lit)

- As shown below, you can see the following data in the channel information: RX Fre[MHz], TX Fre[MHz], CTCSS/ DCS Dec, CTCSS/DCS Enc, Tx Power, channel spacing; 
Cclick the icon to edit specifications.

Note: Since the GM series have only 16 channels, the channel 17 can not be edited.

4. Click Function Setup on the left. You can set the relevant function parameters according to your needs.

5. After the setup is complete, click the icon or Program → Write data to Interphone(R) to write the data to the radio.